# Experiment - 1

Aim: A 3- ø, 230 V, 50 Hz is connected to a 3- ø load of 10kW, 0.9pf lagging. Calculate line current and line voltage for

- When supply is star connected and load is also star connected

- When supply is star connected and load is delta connected

- When supply is delta connected and load is star connected

- When supply is delta connected and load is also delta connected

Code/Model Parameters:

| Parameters | Values |
| --- | --- |
| Voltage | 220 V |
| Frequency | 50 Hz |
| Load | 10 kW |
| Load Power Factor | 0.9 lagging |

MATLAB Code (.m file): Star Star Connection

### Syntax

clear; clc;

close all;

Vph = input("Enter source side phase voltage(star, in volts) : "); P = input("Enter power consumed on load side(star, in watts) : "); pf = input("Enter power factor : ");

Vline = Vph*sqrt(3);

Pph = P/3;

Iph = Pph/(Vph*pf);

fprintf('\nLine current is : %0.3f',Iph); %in star, line and phase current are equal

fprintf('\nLine voltage is : %0.3f',Vline);

fprintf('\nPhase voltage is : %0.3f',Vph);

fprintf('\nPhase curent is : %0.3f',Iph);

### Results

Enter source side phase voltage(star, in volts) : 230

Enter power consumed on load side(star, in watts) : 10000

Enter power factor : 0.9

Line current is : 16.103

Line voltage is : 398.372

Phase voltage is : 230.000

Phase curent is : 16.103

## MATLAB Model (.mdl file): Star Star Connection

Diagram of model

Results

## MATLAB Code (.M file): Star Delta Connection

### Syntax

clear; clc;

close all;

Vph = input("Enter source side phase voltage(star, in volts) : ");

P = input("Enter power consumed on load side(delta, in watts) : ");

pf = input("Enter power factor : ");

Pph = P/3;

Iph = Pph/(Vph*pf);

Iline=Iph*sqrt(3);

fprintf('\nLine current is : %0.3f',Iline);

fprintf('\nLine voltage is : %0.3f',Vph);

fprintf('\nPhase voltage is : %0.3f',Vph);

fprintf('\nPhase curent is : %0.3f',Iph);

### Results

Enter source side phase voltage(star, in volts) : 230

Enter power consumed on load side(delta, in watts) : 10000

Enter power factor : 0.9

Line current is : 27.891
Line voltage is : 230.000
Phase voltage is : 230.000
Phase curent is : 16.103

## MATLAB Model (.mdl file): Star Delta Connection

### Diagram of model

Results

# Experiment - 2

Aim: What is the percentage saving in feeder copper if the line voltage in a 2-wire d.c. system is raised from 200 volts to 400 volts for the same power transmitted over the same distance and having the same power loss?

Code/Model Parameters:

| Parameters | Values |
| --- | --- |
| Initial Voltage | 200 V |
| Final Voltage | 400 V |

MATLAB Code (.m file)

### Syntax

clear; clc;

a = input('Initial Voltage : ');

b = input('New Voltage : ');

s = (1-(a/b)^2)*100; fprintf('Savings is : %0.2f',s);

### Results

Initial Voltage : 200 New Voltage : 400 Savings is : 75.00

## MATLAB Model (.mdl file)

Diagram of model